Norco College ARE25

Santiago High Arch Design 2A/2B

Revit MEP 2017 Demo: Electrical

Students will create an electrical plan of the Retail Store. In order, we will:

Create the building in Revit Architecture.

Use EnergyPro to calculate Lighting needs for spaces

Link our Revit Architecture model into Revit MEP

Place the Lighting Fixtures

Place the switches and outlet

Create Lighting and Power Circuits

Create a Panel Schedule

Create Building per Sketch in Revit Architecture

It should have walls, storefront, ceilings, doors, 3D Text sign, foundation, and tile flooring. No plumbing or electrical fixtures. We will do that in Revit MEP.

Some details:

Double Glass Curtain door

14’ Level 2 (name it Roof)

11’  2X2 ACT Ceilings everywhere but Restroooms

9’ 2X2 ACT Ceilings in Restrooms

Ext walls will have a 4’ parapet, making them 18’ total

Make Roof a Generic 12 to start, at the Roof Level (14’)

5” Slab Foundation with footings

Put ¼” ceramic tile on the floor in all rooms. Offset ¼” to go on top of Foundation slab

Paint on walls. You will have to split some walls and “Split Face” some walls.

Place base molding

Save as: Revit Arch Building for MEP.rvt

Linking the Building into Revit MEP

Generally the architect designs the building, then hands it off to the Mechanical Engineer (you) to do the Mechanical, Electrical, and Plumbing plans. We will try to use that workflow….

Open Revit

New….Browse….Electrical Default….Open..OK

File: Save as Store Electrical 2017.rvt

In Floor Plan 1-Lighting view

Insert

Link Revit

Revit Arch Building for MEP 2017.rvt (note: see next line before OK!)

Position: Auto Origin to Origin…..Open

The building should be there. Discuss Linked Files

Note the new Browser setup

Discuss layout by Discipline

Show Properties….Discipline…Electrical….Plumbing, etc

Show subdiscipline….Lighting…..

Lighting Floor Plan Level 1

VV

Note Lighting, Electrical equipment ON, but Ducts, Air Terminals OFF.

East Elevation Lighting

VV

Note Pipe and plumbing fixtures, ducts, air terminals are OFF but Lighting and Electrical Fixtures are ON.

Get it??

Go to Elevation View. The linked building and the template need to have the Levels aligned.

Drag the levels away from each other.

Note Level 1’s are OK

Rename Level 2 to Roof and move it to the other “Roof” level using the Align command

Save as Store Electrical 2017.rvt

Creating the Electrical System: Leveraging EnergyPro data

Before we start throwing in Lights, Panels, etc. we need to know what the building requires! We created an energy model in EnergyPro to give us load information of the Store.

We are specifying the Lithonia 2ES8P 232 T8, WW232IRLS T8, and the 2ES8P 217 T8 Lighting Fixtures

Lighting Ceiling Plan 1-Ceiling Elec

Systems

Lighting Fixture

Load Family

X:\labs\brown\readonly\Arch Design 2A-2B MEP\Revit MEP Families

2ES8P-2X4

Open

Place on Face (not Ceiling Based)

From EnergyPro data (see above)

Set Lamp to T8 and Apparent Load to 64VA (that’s 64W).	OK

We need:

12 2ES8P 232 T8 lights in the STORE

Load the “Wall Wash” lights

Systems

Lighting Fixture

Load Family

X:\labs\brown\readonly\Arch Design 2A-2B MEP\Revit MEP Families

WW

Open

Place on Face (not Ceiling Based)

30 WW 232IRLS T8 lights along the East and West walls of Store (3’ away). 15 each side.

Now the 2X2 lights for the bathrooms:

Systems

Lighting Fixture

Load Family

X:\labs\brown\readonly\Arch Design 2A-2B MEP\Revit MEP Families

2ES8P-2X2

Open

Place on Face (not Ceiling Based)

2 lights in the Hall

2 lights in the Office

3 lights in the Men’s

3 Lights in the Women’s

Save as: Store Electrical 2017.rvt

Placing Electrical Fixtures

Outlets

Power…Floor Plan….1-Power

Note that lights are ON in this view. No “trick” electrical plans!

Systems

Device

Electrical Fixture

Duplex Receptacle (Electrical…..MEP….Electrical Power….Terminals (should be pre- loaded)

Place per sketch

Place on Vertical Face…use spacebar to flip

Switches

Systems

Device

Lighting

Lighting Switch Single Pole

Set scale to ¼”=1’0” to make viewing easier

Outside Lights

Systems……Lighting Fixtures…….Load

Lighting

MEP

External

Spot Light – Ext

Offset: 6’0”

Wires

Lighting….Ceiling Plan….1-Ceiling Elect

You may want to turn off the Ceiling Pattern to allow easier snapping of wires

Systems

Wire

Arc Wire (Spline is OK too)

The connectors look like circle OSNAPS in the middle of the light fixture. The wires must connect to the lights, or your circuit will not work!

Now we need a Junction box….

Junction box

Systems

Device

Electrical Fixture

Load Family

Electrical

MEP

Electrical Power

Distribution

Junction boxes Load

Offset 12’ which put the Junction box between the ceiling and roof

Place per sketch

A “Junction Box” looks like this.

Connect the Junction Box to the switch. Note the switch has a connector toward the back …on the wall side…of the switch. They must “connect”. It’s like a circle with an X thru it OSNAP. The wire will jump to a different spot on the fixture….that’s OK, it’s connected!

Continue making the circuits per sketch.

We have motion sensors controlling the bathroom lights.

Occupancy Sensors

Systems

Device

Lighting

Load Family

Electrical

MEP

Info and communication

Building Controls

Ceiling Occupancy sensor Low Voltage

Place on Face

There are several ways this wiring diagram could go. You could put the wall wash lights on their own circuit. You could put the 12 store lights on their own circuit. On my diagram, I am splitting the circuits into front of store zone and back of store zone. What is the best strategy???

THIS DRAWING IS WRONG…..outlets NOT to junction box

The lights are wired up. May look something like this:

Hint: Use the tab key to see if the lights are connected! Select one light….hit TAB. See the blue?

Outlets

The outlets need to be connected with wires too. See sketch. See the arrow. That is the “Home Run”. The Home Run is the line from first point of the circuit that goes back to the circuit panel/breaker box.

Read more: http://www.doityourself.com/forum/electrical-ac-dc/247908-home-run-wiring-means-what.html#ixzz38DIZaAhI

Note the Office needs to be on its own circuit because the client wants AFCI protection. (BTW, In California, Dedicated AFCI circuits are required in residential bedrooms).

Note the restroom need GFCI protection by code.

Once the outlets are connected with wires, you will need to create circuits:

Creating an Outlet Circuit

Pick Outlet

Power

Edit Circuit

+ Add to Circuit

Pick the outlets you want on the circuit.

Ignore the warning box.

When all are black, check Finish Editing Circuit

Do that with each outlet circuit….

Creating a Lighting Circuit

Pick front lights (use filter)

Power

Edit Circuit

Add junction box and switch

Finish Editing Circuit

Pick rear lights (use filter)

Power

Edit Circuit

Add junction box and switch

Finish Editing Circuit

Pull wires off junction box, then re-attach to get rid of home run arrows.

Location of Electrical Components:

Switches and Outlets

Systems

Device

Electrical

MEP

Electrical Power

Terminals

Duplex, Light Switches, Weather Proof Receptacles

Thermostats and Sensors

Systems

Device

Lighting

Load Family

Electrical

MEP

Info and communication

Building Controls

Telephone and data

Systems

Device

Lighting

Load Family

Electrical

MEP

Info and communication

Communication

Data Junction box

Telephone junction box

Data outlets

Ethernet

Telephone outlet

Speakers

Security, smoke, fire alarms

Systems

Device

Lighting

Load Family

Electrical

MEP

Info and communication

Fire Alarm

Fire Alarms

Smoke detectors

Security

Alarm

Adding a Breaker Box/Circuit Panel

Systems

Electrical Equipment

Load Family

Electrical

MEP

Electrical Power

Distribution

Lighting and Appliance Panelboard 208V MLO

Choose the 100A type

Place on back wall

Assigning a “Distribution System”

Pick on Panelboard

Distribution System: 120/208 WYE (in Option bar)

Assigning a Circuit to the Panel

Pick on light in front right of Store

Electrical Circuits

Select Panel

Select Panel: 100A, 120V/208V Three Phase, 4 Wires, WYE (in option bar)

Done!

Let’s do another….

Pick a light on the next circuit

Electrical Circuits

Select Panel: 100A, 120V/208V Three Phase, 4 Wires, WYE (in option bar)

Done!

Tagging the Circuits

(not what I mean….)

Annotate……Tag by category

Pick circuit

See the tag? You should have 6 circuits.

Assigning Outlets Circuits

It’s the same process as the lights!

Floor Plan 1-Power

Pick Outlet

Electrical Circuits

Select Panel

Select Panel: 100A, 120V/208V Three Phase, 4 Wires, WYE (in option bar)

Continue this for all the outlet circuits….

Now tag them.

Annotate……Tag by category

Pick outlet circuit

See the tag? You should have about 11 circuits.

Panel Schedule

Now we will create a basic Electrical panel Schedule

Analyze

Panel Schedules

We only have one….

OK

There it is!

Note: See it in Project Browser

Add Space Tags

Save as FirstLast Store Electrical 2014.rvt